## ZPA Setting (Zero Pressure Accumulation)

The MDR is an enable condition. If you set 10 sec (MDR OFF DELAY COLUMN), after 10 sec of MDR running automatically the MDR will cut off.

IF Disable condition MDR is run continuously.

|  | ZPA Setting (Zero Pressure Accumulation) |  |  |
| --- | --- | --- | --- |
| Flags |  | State | Description |
| Mode |  | OFF | ZPA mode Disabled |
|  |  | ON | ZPA mode Enabled, But Other Communication it dose not connect. |
|  |  | Singulation | Bus Mode |
|  |  | GAP Train | Train Mode |
| MDR Direction |  | Clock wise (CW) | Forward Direction |
|  |  | Counter Clock wise (CCW) | Reverse Direction |
| UP Stream IP: |  | IP of the First Network | Enter the address |
| Down Stream IP: |  | IP of the Last Network Card | Enter the address |
| MDR OFF Flag |  | Enable | When Sensor is Connected |
|  |  | Disable | When Sensor Not Connected |
| MDR Off Delay |  | Delay time | Delay time in Seconds |

- Module - I, II, III & IV

- Left Sensor Input 1 (Module-1)

- Right Sensor Input 1 (Module-2)

- Left MDR (Module-1)

- Right MDR (Module-2)

- Ethernet communication

Sir, ZPA mode one Smart roll card not working condition.

2 card above Smart roll card ZPA work condition

ZPA mode clack wise conveyor running.

| Sensor_1 | Sensor_2 | Sensor_3 | Sensor_4 | Modul_1 | Modul_2 | Modul_3 | Modul_4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |

ZPA Mode Network Card IP Address Setting :

Expel : 4 Network card series connection ZPA mode clack wise conveyor running.

IP Adress                                   IP Adress                              IP Adress                              IP Adress

192.168.250.1                          192.168.250.2                    192.168.250.3                       192.168.250.4

DN Stream IP
192.168.250.4

Up Stream IP
192.168.250.3

DN Stream IP
192.168.250.4

Up Stream IP
192.168.250.2

Up Stream IP
192.168.250.1

DN Stream IP
192.168.250.3

Up Stream IP
192.168.250.1

DN Stream IP
192.168.250.2